~3.NAT and PAT Concept Theory:-

1.)In Cisco Packet Tracer, set up a simple network with a router, one internal PC, and one external PC. Assign private IP addresses to the internal PC and public IP addresses to the external PC, then configure static NAT on the router to allow the internal PC to communicate with the external PC.

Static NAT — Internal PC to External PC:

Topology:

Internal Network                    External Network

PC1 ───── Switch ───── R1 ───────────── PC2

|

NAT Router

IP Addressing:

| Device | Interface | IP Address | Type |
| --- | --- | --- | --- |
| PC1 | NIC | 192.168.1.10 | Private |
| R1 | G0/0 | 192.168.1.1 | Private |
| R1 | G0/1 | 203.0.113.1 | Public |
| PC2 | NIC | 203.0.113.10 | Public |

PC1 default gateway:

192.168.1.1

PC2 default gateway:

203.0.113.1

R1 Configuration:

enable

configure terminal

interface gigabitEthernet 0/0

ip address 192.168.1.1 255.255.255.0

ip nat inside

no shutdown

exit

interface gigabitEthernet 0/1

ip address 203.0.113.1 255.255.255.0

ip nat outside

no shutdown

exit

ip nat inside source static 192.168.1.10 203.0.113.100

end

Here:

192.168.1.10 → 203.0.113.100

is the static NAT mapping.

Test:

PC1> ping 203.0.113.10

2.)Configure Port Address Translation (PAT) on a router in Packet Tracer so that two internal PCs with private IPs can access a simulated public web server using a single public IP address.<br><br><em><strong>Hint:</strong> Use the 'ip nat inside source list' command with 'overload' to enable PAT.</em>

Configure PAT for Two Internal PCs:

Topology:

PC1 ─┐

├── Switch ── R1 ─── Web Server

PC2 ─┘

IP Addressing:

| Device | IP Address |
| --- | --- |
| PC1 | 192.168.1.10 |
| PC2 | 192.168.1.20 |
| R1 Inside | 192.168.1.1 |
| R1 Outside | 203.0.113.1 |
| Web Server | 203.0.113.10 |

R1 Configuration:

Configure inside interface:

enable

configure terminal

interface gigabitEthernet 0/0

ip address 192.168.1.1 255.255.255.0

ip nat inside

no shutdown

exit

Configure outside interface:

interface gigabitEthernet 0/1

ip address 203.0.113.1 255.255.255.0

ip nat outside

no shutdown

exit

Create NAT access list:

access-list 1 permit 192.168.1.0 0.0.0.255

Enable PAT:

ip nat inside source list 1 interface gigabitEthernet 0/1 overload

end

How PAT works:

Both PCs use the same public IP:

PC1: 192.168.1.10 ──┐

├── 203.0.113.1

PC2: 192.168.1.20 ──┘

The router uses different port numbers to keep the connections separate.

3.)Test your NAT configuration by sending a ping from the internal PC to the external PC and capture the NAT translation table on the router to verify the address mapping.

Test NAT/PAT and View Translation Table:

From PC1:

ping 203.0.113.10

From PC2:

ping 203.0.113.10

On R1, check the NAT table:

show ip nat translations

You may see entries similar to:

Pro     Inside global      Inside local          Outside local     Outside global

icmp  203.0.113.1:1   192.168.1.10:1    203.0.113.10:1  203.0.113.10:1

icmp  203.0.113.1:2   192.168.1.20:2    203.0.113.10:2  203.0.113.10:2

Also use:

show ip nat statistics

This displays NAT statistics and the number of active translations.

4.)Modify your Packet Tracer topology to add a third internal PC. Update your NAT or PAT configuration so all three internal PCs can access the external network, and verify that each translation appears in the router's NAT table.

Add a Third Internal PC:

Add PC3 to the switch.

PC3 Configuration:

IP Address: 192.168.1.30

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.1.1

The existing PAT configuration already covers the entire:

192.168.1.0/24

because of:

access-list 1 permit 192.168.1.0 0.0.0.255

Therefore, no additional NAT command is required.

Test:

PC3> ping 203.0.113.10

Then check:

R1# show ip nat translations

You should see translations for:

192.168.1.10

192.168.1.20

192.168.1.30

all using the router's public IP:

203.0.113.1

Simple representation:

PC1 192.168.1.10 ──┐

PC2 192.168.1.20 ──┼── R1 ── 203.0.113.1 ── Web Server

PC3 192.168.1.30 ──┘

5.)Explain in your own words how NAT and PAT help apps like Zomato or Flipkart serve millions of users with limited public IP addresses. Give two real-world scenarios where incorrect NAT/PAT setup could break connectivity for users.

NAT and PAT — Real-World Explanation:

How NAT helps Zomato/Flipkart:

NAT (Network Address Translation) converts private IP addresses inside a network into public IP addresses when devices communicate with the Internet.

PAT (Port Address Translation) goes one step further. It allows many private devices to share one public IP address by using different port numbers.

For example:

User 1 → 10.0.0.10 ──┐

User 2 → 10.0.0.11 ──┼── PAT → One Public IP → Internet

User 3 → 10.0.0.12 ──┘

This is useful for large services such as Zomato or Flipkart because huge numbers of devices can communicate with Internet-facing systems without requiring a unique public IPv4 address for every device.

Two real-world problems caused by incorrect NAT/PAT:

1. Online payment/order connection fails:

If NAT or port translation is configured incorrectly, a user's connection may not reach the required server. This could cause problems such as failed API requests, payment errors, or an order page not loading.

2. Application/server becomes unreachable:

If port forwarding or NAT rules are incorrect, traffic coming toward a required public service may be sent to the wrong internal device—or blocked completely. This can make websites, APIs, or other services unavailable.

Important difference:

| NAT | PAT |
| --- | --- |
| Maps private IP to public IP | Maps many private IPs to one public IP |
| Can use one-to-one mapping | Uses port numbers to distinguish connections |
| Public IP requirement is higher | Saves public IPv4 addresses |
| Example: Static NAT | Example: overload |

Key Packet Tracer commands to remember:

ip nat inside

ip nat outside

ip nat inside source static PRIVATE-IP PUBLIC-IP

access-list 1 permit PRIVATE-NETWORK WILDCARD

ip nat inside source list 1 interface G0/1 overload

show ip nat translations

show ip nat statistics

THE END………