~5.Inter-VLAN Routing Concept Theory:-

1.)On a Cisco Packet Tracer or GNS3 simulation, create three VLANs named Insta, Zomato, and Flipkart on a single switch, and assign at least one PC to each VLAN.

Create Three VLANs:

Topology:

PC1 ──┐

│ VLAN 10 - Insta

PC2 ──┤ VLAN 20 - Zomato

│ VLAN 30 - Flipkart

PC3 ──┘

|

Switch

|

| Trunk

|

Router

VLAN Details:

| VLAN ID | VLAN Name | PC | IP Address |
| --- | --- | --- | --- |
| 10 | Insta | PC1 | 192.168.10.10 |
| 20 | Zomato | PC2 | 192.168.20.10 |
| 30 | Flipkart | PC3 | 192.168.30.10 |

Switch Configuration:

enable

configure terminal

vlan 10

name Insta

exit

vlan 20

name Zomato

exit

vlan 30

name Flipkart

exit

Assign PC ports:

Assume:

- PC1 → Fa0/1

- PC2 → Fa0/2

- PC3 → Fa0/3

- Router → Fa0/24

interface fastEthernet 0/1

switchport mode access

switchport access vlan 10

exit

interface fastEthernet 0/2

switchport mode access

switchport access vlan 20

exit

interface fastEthernet 0/3

switchport mode access

switchport access vlan 30

exit

2.)Configure a Router-on-a-Stick setup by creating sub-interfaces (e.g., GigabitEthernet0/0.10, .20, .30) on a simulated router, each with an IP address for the corresponding VLANs Insta, Zomato, and Flipkart.

Configure Router-on-a-Stick:

Connect the router's G0/0 to the switch's Fa0/24.

Router R1:

enable

configure terminal

interface gigabitEthernet 0/0

no shutdown

exit

VLAN 10 — Insta:

interface gigabitEthernet 0/0.10

encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

exit

VLAN 20 — Zomato:

interface gigabitEthernet 0/0.20

encapsulation dot1Q 20

ip address 192.168.20.1 255.255.255.0

exit

VLAN 30 — Flipkart:

interface gigabitEthernet 0/0.30

encapsulation dot1Q 30

ip address 192.168.30.1 255.255.255.0

exit

end

3.)Assign the switch port connected to the router as a trunk port, and verify trunking is active using the show interfaces trunk command.<br><br><em><strong>Hint:</strong> Use the switchport mode trunk command on the relevant interface.</em>

Configure Trunk Port:

On the switch, configure Fa0/24 as the trunk connected to the router.

enable

configure terminal

interface fastEthernet 0/24

switchport mode trunk

exit

Verify VLANs:

show vlan brief

You should see:

10   Insta

20   Zomato

30   Flipkart

Verify trunk:

show interfaces trunk

You should see Fa0/24 listed as a trunk port.

The important concept is:

PC → Access Port → Switch → Trunk → Router

↑

VLAN tags

4.)Test inter-VLAN connectivity by pinging from a PC in the Insta VLAN to a PC in the Zomato VLAN, and explain why the ping succeeds or fails based on your configuration.

Configure PCs and Test Inter-VLAN Communication:

PC1 — Insta:

IP Address:      192.168.10.10

Subnet Mask:     255.255.255.0

Default Gateway: 192.168.10.1

PC2 — Zomato:

IP Address:      192.168.20.10

Subnet Mask:     255.255.255.0

Default Gateway: 192.168.20.1

PC3 — Flipkart:

IP Address:      192.168.30.10

Subnet Mask:     255.255.255.0

Default Gateway: 192.168.30.1

Test Insta → Zomato:

From PC1:

ping 192.168.20.10

Expected result:

Reply from 192.168.20.10

The ping succeeds because:

- PC1 sends traffic to its default gateway 192.168.10.1.

- The switch sends VLAN 10 traffic through the trunk.

- Router sub-interface G0/0.10 receives the VLAN 10 traffic.

- The router routes it to VLAN 20 through G0/0.20.

- The switch delivers the packet to PC2.

If the ping fails:

Check:

show vlan brief

show interfaces trunk

show ip interface brief

Also verify that the PC gateways and router sub-interface IP addresses are correct.

5.)Refactor your Router-on-a-Stick configuration to add a new VLAN called 'Spotify' without disrupting existing VLAN communication, and document the exact commands you used for both the switch and the router.

Add Spotify VLAN Without Disrupting Existing VLANs:

Now add:

VLAN ID: 40

Name: Spotify

Assume PC4 is connected to Fa0/4.

Step 1 — Create Spotify VLAN on Switch:

enable

configure terminal

vlan 40

name Spotify

exit

Step 2 — Assign PC4 to VLAN 40:

interface fastEthernet 0/4

switchport mode access

switchport access vlan 40

exit

Step 3 — Verify:

show vlan brief

You should now have:

10   Insta

20   Zomato

30   Flipkart

40   Spotify

Step 4 — Configure Spotify Router Sub-interface:

On R1:

enable

configure terminal

interface gigabitEthernet 0/0.40

encapsulation dot1Q 40

ip address 192.168.40.1 255.255.255.0

exit

end

The existing sub-interfaces .10, .20 and .30 are not changed.

Step 5 — Configure Spotify PC:

PC4:

IP Address:      192.168.40.10

Subnet Mask:     255.255.255.0

Default Gateway: 192.168.40.1

Test:

PC4> ping 192.168.40.1

Then test existing VLANs:

PC4> ping 192.168.10.10

PC4> ping 192.168.20.10

PC4> ping 192.168.30.10

If routing is configured correctly, these should work.

Final Configuration Summary:

Switch:

enable

configure terminal

vlan 10

name Insta

exit

vlan 20

name Zomato

exit

vlan 30

name Flipkart

exit

vlan 40

name Spotify

exit

interface fa0/1

switchport mode access

switchport access vlan 10

exit

interface fa0/2

switchport mode access

switchport access vlan 20

exit

interface fa0/3

switchport mode access

switchport access vlan 30

exit

interface fa0/4

switchport mode access

switchport access vlan 40

exit

interface fa0/24

switchport mode trunk

exit

Router:

enable

configure terminal

interface g0/0

no shutdown

exit

interface g0/0.10

encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

exit

interface g0/0.20

encapsulation dot1Q 20

ip address 192.168.20.1 255.255.255.0

exit

interface g0/0.30

encapsulation dot1Q 30

ip address 192.168.30.1 255.255.255.0

exit

interface g0/0.40

encapsulation dot1Q 40

ip address 192.168.40.1 255.255.255.0

exit

end

Important verification commands:

show vlan brief

show interfaces trunk

show ip interface brief

show running-config

Conclusion: Router-on-a-Stick allows one physical router interface to route traffic between multiple VLANs by using 802.1Q-tagged sub-interfaces. Adding VLAN 40 only requires a new VLAN, access-port assignment, and router sub-interface, so the existing Insta, Zomato, and Flipkart communication remains unchanged.

THE END……