~4.Weekly Practice and Troubleshooting Practical:-

1.)Use Cisco Packet Tracer to simulate a WAN setup connecting two branch offices via a VPN tunnel and configure NAT on the edge routers so both branches can access the internet.

WAN with VPN + NAT:

Topology:

WAN / Internet

|

+-------+-------+

|               |

R1               R2

Branch 1         Branch 2

|               |

Switch1          Switch2

/    \            /   \

PC1    PC2        PC3   PC

For the lab, use:

| Device | Interface | IP Address |
| --- | --- | --- |
| R1 LAN | G0/0 | 192.168.10.1/24 |
| R1 WAN | G0/1 | 203.0.113.1/30 |
| R2 LAN | G0/0 | 192.168.20.1/24 |
| R2 WAN | G0/1 | 203.0.113.2/30 |
| PC1 | NIC | 192.168.10.10/24 |
| PC2 | NIC | 192.168.10.20/24 |
| PC3 | NIC | 192.168.20.10/24 |

Note: In a real network, the public-facing addresses would be assigned by the ISP. These addresses are being used here only as a Packet Tracer lab example.

R1 basic interfaces:

enable

configure terminal

interface g0/0

ip address 192.168.10.1 255.255.255.0

ip nat inside

no shutdown

exit

interface g0/1

ip address 203.0.113.1 255.255.255.252

ip nat outside

no shutdown

exit

R2 basic interfaces:

enable

configure terminal

interface g0/0

ip address 192.168.20.1 255.255.255.0

ip nat inside

no shutdown

exit

interface g0/1

ip address 203.0.113.2 255.255.255.252

ip nat outside

no shutdown

exit

GRE VPN tunnel:

R1:

interface tunnel 0

ip address 172.16.0.1 255.255.255.252

tunnel source g0/1

tunnel destination 203.0.113.2

no shutdown

exit

ip route 192.168.20.0 255.255.255.0 172.16.0.2

R2:

interface tunnel 0

ip address 172.16.0.2 255.255.255.252

tunnel source g0/1

tunnel destination 203.0.113.1

no shutdown

exit

ip route 192.168.10.0 255.255.255.0 172.16.0.1

Test:

R1# ping 172.16.0.2

Then:

PC1> ping 192.168.20.10

2.)Configure Port Address Translation (PAT) on a router in your simulated WAN so that three devices from the internal network can browse the internet using a single public IP address.<br><br><em><strong>Hint:</strong> Use the 'overload' keyword in your NAT configuration.</em>

PAT for Three Internal Devices:

On R1, create an ACL for the internal network:

access-list 1 permit 192.168.10.0 0.0.0.255

Enable PAT:

ip nat inside source list 1 interface g0/1 overload

Now PC1, PC2 and a third PC can share the same public IP.

PC1 192.168.10.10 ──┐

PC2 192.168.10.20 ──┼── R1 ── 203.0.113.1 ── Internet

PC3 192.168.10.30 ──┘

Test from each PC:

ping <Internet/Web-Server-IP>

Check the translations:

R1# show ip nat translations

Also:

R1# show ip nat statistics

3.)Intentionally misconfigure the VPN tunnel's pre-shared key between two routers in your WAN simulation, then use the 'show crypto isakmp sa' and 'show run' commands to identify and fix the connectivity issue.

Troubleshoot Wrong VPN Pre-Shared Key:

For an IPsec VPN lab, suppose R1 and R2 have different pre-shared keys.

R1:

crypto isakmp policy 10

authentication pre-share

encryption aes

hash sha

group 2

exit

crypto isakmp key cisco123 address 203.0.113.2

Suppose R2 accidentally has:

crypto isakmp key wrongkey address 203.0.113.1

The VPN negotiation will fail because the keys don't match.

Check the problem:

On R1:

show crypto isakmp sa

You may see an incomplete/non-established ISAKMP state instead of a successful QM_IDLE state.

Check the configuration:

show running-config

Look for:

crypto isakmp key

Fix R2:

configure terminal

no crypto isakmp key wrongkey address 203.0.113.1

crypto isakmp key cisco123 address 203.0.113.1

end

Then clear old negotiations if necessary:

clear crypto isakmp sa

Test again:

show crypto isakmp sa

A successful IKE/IPsec setup should show an established security association.

4.)Troubleshoot a scenario where devices in one branch office can ping the internet but cannot access devices in the other branch via the VPN tunnel; document the commands you use and the configuration change that resolves the problem.

Branch-to-Branch VPN Connectivity Problem:

Problem:

Suppose:

Branch 1 PC → Internet       SUCCESS

Branch 1 PC → Branch 2 PC    FAIL

This means NAT/Internet connectivity can be working while the VPN routing or VPN policy is incorrect.

Step 1 — Check interfaces:

show ip interface brief

Make sure the relevant interfaces are up/up.

Step 2 — Check routing:

On R1:

show ip route

Check for:

192.168.20.0/24

If it is missing, add:

ip route 192.168.20.0 255.255.255.0 172.16.0.2

On R2:

show ip route

Check for:

192.168.10.0/24

If missing:

ip route 192.168.10.0 255.255.255.0 172.16.0.1

Step 3 — Check tunnel:

show interfaces tunnel 0

The tunnel should be:

Tunnel0 is up, line protocol is up

Step 4 — Test tunnel:

From R1:

ping 172.16.0.2

Step 5 — Test remote LAN:

ping 192.168.20.10

Step 6 — Check NAT:

show ip nat translations

If the branch-to-branch traffic is accidentally being NATed, configure a NAT exemption/no-NAT rule appropriate to the platform and topology.

Useful troubleshooting commands:

show ip interface brief

show ip route

show interfaces tunnel 0

show ip nat translations

show crypto isakmp sa

show crypto ipsec sa

For deeper troubleshooting, where supported:

debug crypto isakmp

debug crypto ipsec

After debugging, turn it off:

undebug all

Configuration change that resolves the example problem:
The missing route to the remote branch was added through the GRE tunnel:

ip route 192.168.20.0 255.255.255.0 172.16.0.2

5.)Take the WAN, VPN, and NAT scenario you built and swap your configuration file with a classmate; review their setup, find one misconfiguration, and write a short note on how you identified and fixed it.<br><br><em><strong>Constraint:</strong> Focus on using 'show' and 'debug' commands for troubleshooting rather than guessing the issue.</em>

Classmate Configuration Review:

For this practical, use your classmate's .pkt file and troubleshoot it using show/debug commands instead of guessing.

Review procedure:

1. Check interfaces:

show ip interface brief

2. Check routing:

show ip route

3. Check NAT:

show ip nat translations

show ip nat statistics

4. Check VPN:

show crypto isakmp sa

show crypto ipsec sa

5. Check configuration:

show running-config

6. If necessary, use debugging:

debug crypto isakmp

debug crypto ipsec

Then test the connection and observe the messages.

Finally:

undebug all

short note for submission:

I reviewed my classmate's WAN, VPN, and NAT Packet Tracer configuration. I first used show ip interface brief and show ip route to check the interfaces and routing table. I then used show crypto isakmp sa and show crypto ipsec sa to check the VPN status.

I found that the VPN configuration had an incorrect pre-shared key on one router. I confirmed the problem by checking the running configuration and observing the VPN negotiation with the debug commands.

I corrected the pre-shared key so that it matched on both routers. After clearing the old security association, I tested the connection again. The VPN became established and the branch-to-branch ping was successful.

This troubleshooting showed that show and debug commands can be used to identify configuration problems instead of changing settings by guesswork.

⭐ Most important commands for the practical:

show ip interface brief

show ip route

show running-config

show ip nat translations

show ip nat statistics

show interfaces tunnel 0

show crypto isakmp sa

show crypto ipsec sa

debug crypto isakmp

debug crypto ipsec

undebug all

THE END…….