~6.Layer 3 Switch Inter-VLAN Routing Thoery:-

1.)On a Cisco Layer 3 switch, create three VLANs (e.g., VLAN 10 for 'Sales', VLAN 20 for 'Support', VLAN 30 for 'Marketing') and assign at least one interface to each VLAN.

Create Three VLANs:

Topology:

PC1 ── Fa0/1 ─┐

│ VLAN 10 - Sales

PC2 ── Fa0/2 ─┤ VLAN 20 - Support

│ VLAN 30 - Marketing

PC3 ── Fa0/3 ─┘

|

Layer 3 Switch

VLAN and IP Plan:

| VLAN | Name | SVI IP | PC IP |
| --- | --- | --- | --- |
| 10 | Sales | 192.168.10.1/24 | 192.168.10.10 |
| 20 | Support | 192.168.20.1/24 | 192.168.20.10 |
| 30 | Marketing | 192.168.30.1/24 | 192.168.30.10 |

Create VLANs on the Layer 3 Switch:

enable

configure terminal

vlan 10

name Sales

exit

vlan 20

name Support

exit

vlan 30

name Marketing

exit

Assign ports:

Assume:

- PC1 → Fa0/1

- PC2 → Fa0/2

- PC3 → Fa0/3

interface fastEthernet 0/1

switchport mode access

switchport access vlan 10

exit

interface fastEthernet 0/2

switchport mode access

switchport access vlan 20

exit

interface fastEthernet 0/3

switchport mode access

switchport access vlan 30

exit

Verify:

show vlan brief

2.)Configure Switch Virtual Interfaces (SVIs) for each VLAN you created by assigning them unique IP addresses (e.g., 192.168.10.1/24 for VLAN 10, etc.) and ensure each SVI is in 'up/up' state.<br><br><em><strong>Hint:</strong> Use the 'interface vlan' command to create SVIs and 'no shutdown' to enable them.</em>

Configure SVIs:

Now create a virtual interface for each VLAN.

VLAN 10 — Sales:

interface vlan 10

ip address 192.168.10.1 255.255.255.0

no shutdown

exit

VLAN 20 — Support:

interface vlan 20

ip address 192.168.20.1 255.255.255.0

no shutdown

exit

VLAN 30 — Marketing:

interface vlan 30

ip address 192.168.30.1 255.255.255.0

no shutdown

exit

Important: An SVI may not become up/up merely from no shutdown. The corresponding VLAN must exist and normally have at least one active switch port in that VLAN.

Verify:

show ip interface brief

Expected:

Interface              IP-Address       Status    Protocol

Vlan10                 192.168.10.1     up        up

Vlan20                 192.168.20.1     up        up

Vlan30                 192.168.30.1     up        up

3.)Enable IP routing on the Layer 3 switch so that devices in different VLANs can communicate with each other without using an external router.<br><br><em><strong>Hint:</strong> Use the 'ip routing' global configuration command.</em>

Enable Layer 3 Routing:

This is the key difference between a normal Layer 2 switch and a Layer 3 switch.

enable

configure terminal

ip routing

end

Verify:

show ip route

You should see connected routes similar to:

C    192.168.10.0/24 is directly connected, Vlan10

C    192.168.20.0/24 is directly connected, Vlan20

C    192.168.30.0/24 is directly connected, Vlan30

The Layer 3 switch can now route directly between the VLANs.

4.)Verify inter-VLAN routing by connecting two PCs (one in VLAN 10 and one in VLAN 20) and pinging between them. Capture and submit the ping results to show successful communication.

Test Inter-VLAN Routing:

PC1 — Sales

IP Address:      192.168.10.10

Subnet Mask:     255.255.255.0

Default Gateway: 192.168.10.1

PC2 — Support:

IP Address:      192.168.20.10

Subnet Mask:     255.255.255.0

Default Gateway: 192.168.20.1

Test from PC1:

PC1> ping 192.168.20.10

Expected result:

Reply from 192.168.20.10: bytes=32 time<1ms TTL=127

Reply from 192.168.20.10: bytes=32 time<1ms TTL=127

Reply from 192.168.20.10: bytes=32 time<1ms TTL=127

Reply from 192.168.20.10: bytes=32 time<1ms TTL=127

Why does it work?

PC1

192.168.10.10

|

↓

VLAN 10 / SVI 192.168.10.1

|

↓

Layer 3 Switch

|

↓

VLAN 20 / SVI 192.168.20.1

|

↓

PC2

192.168.20.10

The Layer 3 switch performs the routing between VLAN 10 and VLAN 20. No external router is required.

5.)Refactor your VLAN and SVI configuration so that VLAN 30 uses a different subnet mask (e.g., /25 instead of /24). Explain what changes you made and how this affects communication with the other VLANs.

Change VLAN 30 to /25:

Initially VLAN 30 was:

192.168.30.0/24

Now change it to:

192.168.30.0/25

/25 subnet mask

255.255.255.128

Change the SVI:

enable

configure terminal

interface vlan 30

ip address 192.168.30.1 255.255.255.128

no shutdown

exit

end

Update Marketing PC:

PC3 must also use the new mask:

IP Address:      192.168.30.10

Subnet Mask:     255.255.255.128

Default Gateway: 192.168.30.1

Verify:

show ip interface brief

show ip route

You should see VLAN 30 as:

192.168.30.0/25

Effect of Changing /24 → /25

| Feature | /24 | /25 |
| --- | --- | --- |
| Subnet mask | 255.255.255.0 | 255.255.255.128 |
| Total addresses | 256 | 128 |
| Usable host addresses | 254 | 126 |
| Network | 192.168.30.0 | 192.168.30.0 |
| Broadcast | 192.168.30.255 | 192.168.30.127 |

The other VLANs do not need to change. VLAN 10 remains 192.168.10.0/24 and VLAN 20 remains 192.168.20.0/24.

The Layer 3 switch can still route between all three VLANs because each VLAN has its own separate IP subnet.

Important: Every device in VLAN 30 must use a compatible /25 mask. If the SVI is /25 but a Marketing PC incorrectly remains /24, communication can behave unexpectedly, especially for addresses near the subnet boundary.

Final verification commands:

show vlan brief

show ip interface brief

show ip route

show running-config

⭐ Key commands to remember:

vlan 10

name Sales

interface vlan 10

ip address 192.168.10.1 255.255.255.0

no shutdown

ip routing

Conclusion: A Layer 3 switch uses SVIs as gateways for VLANs and ip routing to provide inter-VLAN communication directly. Changing Marketing from /24 to /25 reduces its available host range but does not affect routing between Sales, Support, and Marketing.

THE END…..