~2. Introduction to VPNs Theory:-

1.)In Cisco Packet Tracer, create two separate networks (representing two branch offices) using two routers and configure basic IP addressing so that each network can ping its own devices.

Create Two Branch Networks:

Topology:

Branch 1                                  Branch 2

PC1 ── Switch1 ── R1 ===== R2 ── Switch2 ── PC2

WAN

IP Addressing:

| Device | Interface | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| PC1 | NIC | 192.168.10.10 | 255.255.255.0 |
| R1 | G0/0 | 192.168.10.1 | 255.255.255.0 |
| R1 | G0/1 | 10.0.0.1 | 255.255.255.252 |
| R2 | G0/1 | 10.0.0.2 | 255.255.255.252 |
| R2 | G0/0 | 192.168.20.1 | 255.255.255.0 |
| PC2 | NIC | 192.168.20.10 | 255.255.255.0 |

R1 Configuration:

enable

configure terminal

interface gigabitEthernet 0/0

ip address 192.168.10.1 255.255.255.0

no shutdown

exit

interface gigabitEthernet 0/1

ip address 10.0.0.1 255.255.255.252

no shutdown

exit

R2 Configuration:

enable

configure terminal

interface gigabitEthernet 0/0

ip address 192.168.20.1 255.255.255.0

no shutdown

exit

interface gigabitEthernet 0/1

ip address 10.0.0.2 255.255.255.252

no shutdown

exit

Set the PCs:

PC1:

IP: 192.168.10.10

Mask: 255.255.255.0

Gateway: 192.168.10.1

PC2:

IP: 192.168.20.10

Mask: 255.255.255.0

Gateway: 192.168.20.1

2.)Configure a basic site-to-site VPN between the two routers in Packet Tracer using GRE tunnel interfaces and assign tunnel IP addresses so that PCs from one branch can ping PCs in the other branch.<br><br><em><strong>Hint:</strong> Use the 'interface tunnel' command on both routers and set the tunnel source and destination to the routers' public-facing interfaces.</em>

Site-to-Site VPN Using GRE Tunnel:

Create a GRE tunnel between R1 and R2.

R1:

enable

configure terminal

interface tunnel 0

ip address 172.16.0.1 255.255.255.252

tunnel source gigabitEthernet 0/1

tunnel destination 10.0.0.2

no shutdown

exit

ip route 192.168.20.0 255.255.255.0 172.16.0.2

end

R2:

enable

configure terminal

interface tunnel 0

ip address 172.16.0.2 255.255.255.252

tunnel source gigabitEthernet 0/1

tunnel destination 10.0.0.1

no shutdown

exit

ip route 192.168.10.0 255.255.255.0 172.16.0.1

end

Verify the tunnel:

On R1:

show interfaces tunnel 0

You should see the tunnel interface as up/up.

Test from R1:

ping 172.16.0.2

Then from PC1:

ping 192.168.20.10

If successful, Branch 1 PC can communicate with Branch 2 PC through the GRE tunnel.

Important: GRE by itself is a tunnel/encapsulation mechanism; it does not encrypt traffic. A true encrypted site-to-site VPN normally combines GRE with IPsec.

3.)Simulate a remote access VPN scenario in Packet Tracer by configuring a VPN client PC to connect securely to one of the branch routers using the built-in VPN client feature.<br><br><em><strong>Hint:</strong> Use the VPN tab on the PC and set the correct server IP, username, and password.</em>

Remote Access VPN:

For the remote-access part, use a VPN-capable router/device and VPN Client feature available in your Packet Tracer version.

The basic concept is:

Remote PC

|

| VPN connection

↓

Branch Router

|

Branch Network

Example VPN client settings:

On the remote PC:

Desktop → VPN:

Enter the VPN information provided by your router configuration:

Server IP: <Branch Router VPN Server IP>

Username: remoteuser

Password: cisco123

Then click Connect.

After the VPN connection is established, test access to the branch network:

ping 192.168.10.10

4.)List and explain the main differences between site-to-site VPN and remote access VPN based on your Packet Tracer configurations, including at least two advantages and disadvantages of each approach.

Site-to-Site VPN vs Remote Access VPN:

| Feature | Site-to-Site VPN | Remote Access VPN |
| --- | --- | --- |
| Purpose | Connects two complete networks | Connects individual users/devices |
| Example | Ahmedabad branch ↔ Surat branch | Employee ↔ company network |
| Configuration | Configured on routers/firewalls | Configured on client + VPN server |
| Users | Many users at each branch | Usually individual remote users |
| Tunnel | Router-to-router | PC/client-to-router/server |
| Maintenance | Centralized at branch routers | Client configuration is also required |

Site-to-Site VPN Advantages:

- Connects entire networks — All devices in both branches can communicate without configuring a VPN separately on every PC.

- Good for permanent connections — Useful for connecting company branches continuously.

Site-to-Site VPN Disadvantages:

- Router configuration is more complex — Both routers need correct tunnel and routing configuration.

- More infrastructure required — Usually requires VPN-capable routers/firewalls at both locations.

Remote Access VPN Advantages:

- Supports remote users — Employees can connect to the company network from outside the office.

- Flexible — Users can connect from different locations using a VPN client.

Remote Access VPN Disadvantages:

- Each client needs configuration — Username, password, VPN settings, etc. must be managed.

- Performance depends on the user's connection — A slow or unstable Internet connection can affect VPN access.

5.)Document and submit the step-by-step configuration commands you used to set up the site-to-site VPN in Packet Tracer, including interface, tunnel, and routing commands.

Step-by-Step Site-to-Site VPN Configuration:

Step 1 — Configure R1 LAN:

enable

configure terminal

interface gigabitEthernet 0/0

ip address 192.168.10.1 255.255.255.0

no shutdown

exit

Step 2 — Configure R1 WAN:

interface gigabitEthernet 0/1

ip address 10.0.0.1 255.255.255.252

no shutdown

exit

Step 3 — Configure R2 LAN:

enable

configure terminal

interface gigabitEthernet 0/0

ip address 192.168.20.1 255.255.255.0

no shutdown

exit

Step 4 — Configure R2 WAN:

interface gigabitEthernet 0/1

ip address 10.0.0.2 255.255.255.252

no shutdown

exit

Step 5 — Create GRE Tunnel on R1:

interface tunnel 0

ip address 172.16.0.1 255.255.255.252

tunnel source gigabitEthernet 0/1

tunnel destination 10.0.0.2

no shutdown

exit

Step 6 — Create GRE Tunnel on R2:

interface tunnel 0

ip address 172.16.0.2 255.255.255.252

tunnel source gigabitEthernet 0/1

tunnel destination 10.0.0.1

no shutdown

exit

Step 7 — Add Routes on R1:

ip route 192.168.20.0 255.255.255.0 172.16.0.2

Step 8 — Add Routes on R2:

ip route 192.168.10.0 255.255.255.0 172.16.0.1

Step 9 — Verify:

show ip interface brief

show interfaces tunnel 0

show ip route

Test tunnel:

R1# ping 172.16.0.2

Test branch-to-branch communication:

PC1> ping 192.168.20.10

Expected result: PC1 should successfully ping PC2 through the GRE tunnel.

THE END…….